SECRET

FUNCTION INTRODUCTION

| Function management |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Function Name: |  | Electronic Stability Control |  |  |  |
| Function Mapping: |  | Driver assistance, autonomous driving, and chassis management |  |  |  |
| Function: |  |  |  | Function-ID: |  |
|  | Name |  | Roler | Date | signature |
| Create/Modify: |  |  |  |  |  |
| check: |  |  |  |  |  |
| Approve OMK: |  |  |  |  |  |

1	Function Introduction	3

1.1	Function description	3

1.2	Function partner list	3

1.3	Function action mode diagram	4

2	System Architecture	5

2.1	Function architecture	5

2.2	Sub-function list	5

2.3	Description of function interface	6

3	Basis function specificaiton	7

3.1	Description of the function status	7

3.2	Logical signals of function	8

3.3	Base Parameters of function	9

3.4	State interpretation and diagram	10

3.5	Diagnostic requirement	10

4	Major Additional Function	11

4.1	Active Roll Mitigation    	11

5	Special States Detect	12

5.1	Specific driving scenarios detection	13

5.2	Performance Mode	13

6	Functional safty	13

6.1	Hazard and Risk analysis	13

6.2	Functional safety concept	13

7	Regulatory Requirements	13

8	Function Dependencies	14

8.1	Internal	14

8.2	Extra	14

8.3    Self-checking strategy...........................................................................................................................15

9	Special Performance Requirements	16

10	Appendix A	17

10.1	Change History	17

# Function Introduction

## Function description

ESC (Electronic Stability Control) is an active safety system. It can be used interchangeably with

VDC,VSC,AYC. The intent of this subsystem is to prevent your vehicle from deviating from the intended path of the driver when the car is rotating in some fashion. This rotation can come from the driver’s steering wheel inputs, road conditions (curvature and banking), throttle input or brake input.

Sometimes, for whatever reason, the vehicles’ actual path does not match the path that the driver

intends, which results in a loss of vehicle control to a large or a small degree. When the ESC system activates, it selectively applies brake pressure to one or more wheels at a time to cause the car to rotate and steer along the intended path of travel.

Figure1. Comparison with or without ESC

Electronic Stability Control is designed to assist the driver in critical driving situations. Entering a bend too quickly, rapid change of surface adhesion or a suddenly occurring obstacle are still dangerous driving situations, but ESC makes vehicle reaction predictable and easier to handle.

## Function partner list

- iBRS

- IMU

- ESC switch

- Wheel Speed Sensor

- LDCU

- EPS

## Function action mode diagram

ESC is used to correct for oversteer and understeer situations.  Below is an illustration of

oversteer and understeer with the results.

Oversteer ESC Control          Understeer ESC Control

Figure2. ESC functions control strategy

For the hydraulic unit, the system build the pressure to control the specific wheel as the request.

Figure3. Hydraulic Schematic

# System Architecture

## Function architecture

ESC controls mainly the vehicle lateral dynamics. This is done by comparing the desired vehicle

motion (driver demand) to the actual vehicle motion. Therefore ESC needs additional sensor

information of yaw rate, lateral acceleration, steering angle and pressure sensor.

Whenever ESC becomes active it will control the vehicle motion in a way to best follow drivers

demand within the restrictions of physics. This can be reached via the interfaces to engine

and/or brake actuator.

Figure 4. ESC system architecture

## Sub-function list

The ESC subsystem includes the below features that will be discussed in this document, but it

can be reduced at the highest level into four separate slip control sub-functions listed as follows:

- Oversteer control

- Understeer control

## Description of function interface

### System dependency (Signals Interaction)

The following table lists the interactive signals for ESC function. The names in the table are from VW CMP21D CEA Comms matrix “VW_CMP21D_CAN_MATRIX20250414 ”.

| Signal Name | Sender | Tx/Rx | Cycle | Unit | Range | ASIL Level | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LDCU_AccPedalSig | LDCU | Rx | 10ms | % | 0-100 | ASIL C | Accelerator pedal position signal |
| LDCU_AccPedalSigVD | LDCU | Rx | 10ms | Enum | 0x0:Not valid; 0x1:Valid; | ASIL C | LDCU_Accelerate Pedal Signal Valid |
| CDCU_ESC_PATASw | CDCU | Rx | 100ms | Enum | 0x0:No command; 0x1:Normal; 0x2:Sport; 0x3:Off; | QM | ESC_OFF SW |
| IPB_VehLongAccel | IMU | Rx(Private) | 10ms | g | -1.73 – 1.73 | ASIL B (D) | Longitudinal Acceleration |
| IPB_VehLongAccelVD | IMU | Rx(Private) | 10ms | Enum | 0x0:Not valid; 0x1:valid; | ASIL B (D) | The valid of longitudinal acceleration |
| IPB_VehLateralAccel | IMU | Rx(Private) | 10ms | g | -1.73 – 1.73 | ASIL B (D) | Lateral Acceleration |
| IPB_VehLateralAccelVD | IMU | Rx(Private) | 10ms | Enum | 0x0:Not valid; 0x1:valid; | ASIL B (D) | The valid of lateral acceleration |
| IPB_YAWRate | IMU | Rx(Private) | 10ms | degps | -93 - 93 | ASIL B (D) | Yaw Rate |
| IPB_YAWRateVD | IMU | Rx(Private) | 10ms | Enum | 0x0:Not valid; 0x1:valid; | ASIL B (D) | The valid of YawRate |
| EPS_SteeringAngle | EPS | Rx | 10ms | deg | -780-779.9 | ASIL B | Steering Angle |
| EPS_SteeringAngleVD | EPS | Rx | 10ms | Boolean | 0x0:Not Valid; 0x1:Valid; | ASIL B | The valid of Steering Angle |
| LDCU_RWheelCurTorq | LDCU | Rx | 10ms | Nm | -6000 - 6000 | ASIL B | LDCU_Rear Wheel Peak Torque Maximum |
| LDCU_FWheelCurTorq | LDCU | Rx | 10ms | Nm | -6000 - 6000 | ASIL B | LDCU_Front Wheel Peak Torque Minimun |
| LDCU_RWheelDriverTorqReq | LDCU | Rx | 10ms | Nm | -6000 - 6000 | ASIL C | LDCU_Rear Wheel Driver Torque Request |
| LDCU_FWheelDriverTorqReq | LDCU | Rx | 10ms | Nm | -6000 - 6000 | ASIL C | LDCU_Front Wheel Driver Torque Request |
| LDCU_GearLev | LDCU | Rx | 10ms | Enum | 0x0:Invalid; 0x1:D gear; 0x2:N gear; 0x3:Rgear; 0x4:P gear; 0x5-0x7:Reserved | ASIL B | LDCU_Gear Level |
| IPB_VDCActSt | iBRS | Tx | 20ms | Enum | 0x0:Not active; 0x1:active; | ASIL B | Indicates VDC in active or not |
| IPB_ESPFault | iBRS | Tx | 20ms | Enum | 0x0:Not fault; 0x1:Fault; | ASIL B | IPB_ESP Fault |
| IPB_ARPActSt | iBRS | Tx | 20ms | Enum | 0x0:Not active; 0x1:Active; | ASIL B | Indication that EBP system currently performs a brake intervention |
| IPB_LDCU_RWheelTorqDecFast | iBRS | Tx | 10ms | Nm | -6000 - 6000 | ASIL C (D) | This torque is VCU combustion(indicated) decreased target torque (fast) |
| IPB_LDCU_FWheelTorqDecFast | iBRS | Tx | 10ms | Nm | -6000 - 6000 | ASIL C (D) | This torque is VCU combustion(indicated) decreased target torque (fast) |
| IPB_BrkLightOnReq | iBRS | Tx | 20ms | Boolean | 0x0:Off; 0x1:On; | ASIL B | Break Light On Request |
| IPB_BrkLightOnReq_E2E | iBRS | Tx | 20ms | Boolean | 0x0:Off; 0x1:On; | ASIL B | Break Light On Request,E2E |

### Hardware Interface

| Name | Type |
| --- | --- |
| KL15 | Hardwired |
| KL30 | Hardwired |
| Left front wheel speed sensor | intelligent |
| Right front wheel speed sensor | intelligent |
| Left rear wheel speed sensor | intelligent |
| Right rear wheel speed sensor | intelligent |
| IPB CAN(H、L) | Hardwired |

# Basis function specification

## Description of the function status

ESC system that uses independent brake and powertrain control to keep the vehicle as close to its intended path as possible, as dictated by sensor inputs from steering angle, vehicle yaw, vehicle lateral acceleration, and a master cylinder pressure sensor.

As shown in the pictures, oversteer (also known as fishtailing, skidding or spinning out) happens when the vehicle rotates more than the driver intends it to.

Understeer (also known as plowing or pushing) happens when the driver turn the wheel and the car

does not turn as much as the driver would like.

To correct for Oversteer, ESC applies pressure to the outside wheels. If you are negotiating a Left-hand turn and the car starts spinning out, the ESC would apply brake pressure to the right front

to bring the car back on path.

Left turn& Oversteer:

Conversely, for Understeer, ESC applies pressure to the inside rear wheel to force the car to rotate

about the center of gravity and to turn the vehicle

Left turn &Understeer:

Figure 5. ESC Schematic Diagram Of The Work

## Logical signals of function

The conditions of ESC enable and activation should meet the following conditions:

- The enable conditions of ESC function:

- The feature is not degraded by fault

And

- Roller bench mode is inactive

And

- LDCU_EVSysReadySt is ready

- The disable conditions of ESC function:

- The feature is degraded by fault

OR

- Roller bench mode is active

OR

-  LDCU_EVSysReadySt is not ready

- The activation conditions of ESC function:

- Driving Mode is not switched to ESC OFF

And

- Vehicle speed is above the threshold TBD1

And

- Driving direction is forward

And

- The deviation between driver intended yaw rate and vehicle yaw rate is greater than the threshold TBD2 (OS) OR The deviation between driver intended yaw rate and vehicle yaw rate is greater than vehicle understeer yaw rate threshold TBD3 (US)

- The deactivation conditions of ESC function:

- Driving Mode is switched to ESC OFF

OR

- Vehicle speed is below the threshold TBD4

OR

- Driving direction is not forward

OR

- The deviation between driver intended yaw rate and vehicle yaw rate is not greater than the threshold TBD5 (OS) AND The deviation between driver intended yaw rate and vehicle yaw rate is not greater than vehicle understeer yaw rate threshold TBD6 (US)

## Base Parameters of function

This table defines the required basic threshold parameters in the function logics. These are all

base values for a specific platform or vehicle. The thresholds are available to be tuned and verified

on the test vehicles during development.

| Condition |  | Description | Remarks |
| --- | --- | --- | --- |
| Enter Condition (AND) | Vehicle Speed | > TBD1 | Tunable |
|  | Delta yaw rate | > TBD2@Oversteer ; > TBD3@Understeer | Tunable |
| Exit Condition (OR) | Vehicle Speed | < TBD4 | Tunable |
|  | Delta yaw rate | ≤ TBD5@Oversteer ; ≤ TBD6@Understeer | Tunable |

## State interpretation and diagram

When ESC is functioning, the state including:

| Function States | Description |
| --- | --- |
|  | YSC_NORMAL |
|  | YSC_ENGINE_ONLY |
|  | YSC_BRAKE_ONLY |
|  | YSC_INHIBIT |

## Diagnostic requirements

When there is a system malfunction (iBRS or vehicle) that affects the ability for ESC to run normal control, then the fault manager will request that ESC enter a special control mode. These modes are reduced functionality. If the fault occurs during ESC control, a graceful shutdown must occur before the ESC is armed in one of the special control modes. This transition is handled via the partial system shutdown logic.

### Special Control Modes

ESC Degradation Control Modes：

| Control Modes | Description |
| --- | --- |
| CM_YSC_NORMAL | - ESC function is normal |
| CM_YSC_INHIBIT | - Inhibit ESC |

### Partial System Shutdown Modes

If a failure occurs while ESC is already active then in most cases the ESC will enter a Partial System Shutdown (PSSD) mode.  It depends on the particular fault and the current status of the ESC control, but a given channel could be requested to run one of the following modes:

- PSSD_YSC_NORMAL

- PSSD_YSC_RAMP_DOWN

- PSSD_ YSC _INHIBIT

# Major Additional Function

## Active Roll Mitigation

### Description of ARM

The Active Roll Mitigation (ARM) function is designed to prevent vehicle rollover by brake

and engine intervention. Dynamic roll over as well as steady state roll over are considered.

ARM relies on early and fast pressure commands to reduce energy in critical situations.

ARM OFF

ARM ON

  - Active Roll Mitigation Control is an active safety system.

  - Detect impending rollover conditions

  - Prevent on-road vehicle roll over situations induced by excessive steering maneuvers.

  - This is done by applying of individual brakes and engine intervention

Figure6. Comparison with or without ARM

### Control Strategy

Main goal of ARM function is to prevent vehicle rollover. ARM algorithm evaluates the vehicle

conditions, determines whether an impending rollover condition exists, and determines

whether to intervene. When ARM activates, it provide pressure and slip commands to the

appropriate wheels in an attempt to prevent rollover by reducing the vehicle speed and

energy.

This Fig. 7, shows the high level signal flow and system architecture environment

of ARM in the Electronic Stability Control (ESC).

Figure 7. System Architecture Environment of ARM in ESC

In ARM system, ARM is designed to reduce roll instability due to fast steering input: fishhook,

double lane change, etc. Ay Limiter (Control Algorithm that limit the Lateral Acceleration of the

Vehicle) is designed reduce roll instability due slow steering input: constant radius, reducing

radius, etc.

- ARM and Ay Limiter commands are arbitrated inside the ARM module

  - ARM has higher priority than Ay Limiter

- The ARM output commands are passed to the ESC module

- The Arbitrator selects whether ARM or ESC is given control

  - Control is given to the system with the larger control request

  - Brake Pressure and Wheel Slip are considered

- Wheel slip is managed by the ESC wheel slip regulator (WSR) for both ARM and ESC

### Test Maneuvers

Sine with D-well, fishhook, double line change are applicable for ARM validation. The test should consider the vehicle’s weight or load changes.

# Special States Detect

## Specific driving scenarios detection

The ESC function detects special scenarios such as banked bend, trailers, and lane changes,

and proactively desensitize the US and OS threshold calculations.

## Performance Mode （CEA2.0 requires）

ESC can change calibrations when the driver selects a different vehicle mode (tour, sport, etc.).

These changes can help change the performance character of the vehicle.

The details will be defined in CEA2.0 performance mode requirement.

# Functional safety

## Hazard and Risk analysis

This part will be detailed in HARA. Refer to the HARA documents provided by VW.

## Functional safety concept

This part will be detailed in FSC. Refer to the FSC documents provided by VW.

### Degradation Mode

Refer to the failsafe manual, which defines details and cases. The failsafe manual should be updated during each software release

# Regulatory Requirements

The ESC performance should meet the following regional regulatory requirements list in the table.

| Region | Document | Title | Version |
| --- | --- | --- | --- |
| EU | ECE R140 | Uniform provisions concerning the approval of passenger cars with regard to Electronic Stability Control (ESC) Systems |  |
| CN | GB/T30677 | Performance requirements and testing methods for electronic stability control system (ESC) for light vehicles |  |
| North America | FMVSS 126 | Electronic stability control systems for light vehicles |  |

# Function Dependencies

| External systems | Description | Responsible |
| --- | --- | --- |
| LDCU | Vehicle Info | LDCU |
| IMU | Accelaration and Yaw Single | EK |
| EPS | Steering ability | EF |
| Foundation Braking | Calipers, discs, tubing | EF |

## Internal

N/A

## Extra

### ESC With HMI：

The function at active or fault information of ESC needs to be displayed by HMI in a timely and

correct manner. The function disable or enable by switch signal sending by HMI via CAN bus.

### ESC Switch Strategy: One virtual switch proposal (sharing with TCS).

- This solution is used for customer vehicle with only one switch signal for ESC (&TCS) OFF, in this case, when switch off, TCS&ESC function are disabled, then switch on, TCS&ESC function are enabled again.

- No remember function, ESC always be enabled for every wake up

- When the driver's side door is opened and TCS&ESC is switched OFF, iBRS shall enable TCS and ESC functions

### ESC torque response requirements

The torque response time requirement to powertrain, should meet the below target:

- Steady response time（t_dead+t_stat):

1) EV Mode ＜ 60ms

2) ICE Mode ＜ 80ms

- Torque overshoot:

Tq_overshoot ＜10%（above 150NM）

Tq_overshoot ＜5%  (below 150NM)

### Sensor Zeroing

Yaw Rate, SWA, Lat Accel, MCP, Long Accel all dynamically “zeroed”. Zeroing communicates

zeroing status to the ESC controller, gains and deadbands adjusted accordingly

## 8.3 Self-checking strategy

After KL15 on, the system will send the default value until self check complete. The HMI will illuminate the lamps and keep on 3s.

## 8.3.1 Function OFF/Active Display Strategy

| Name | Symbol | KL15 Status | HMI Strategy |
| --- | --- | --- | --- |
| ESC Active Lamp |  | ON | -与TCS Active Lamp共用 仪表显示策略： 如 果 收 到（IPB_ESPFault = 0x0:NotFault且IPB_VDCActSt=0x1:active）或（IPB_ESPFault = 0x0:Not Fault且IPB_TCSActSt = 0x1:active），则指示灯按 2Hz 频率进行闪烁。 异常处理： 如果收不到IPB_VDCActSt或IPB_TCSActSt 信号(按通用策略 10 帧认为丢失)，则认为Not active。 备注： N/A |
| ESC OFF Lamp |  | ON | -与TCS OFF Lamp共用 仪表显示策略： 1. 如果收到 IPB_ESPPATA ≠ 0x2:off，不点亮指示灯； 2. 如果收到 IPB_ESPPATA = 0x2:off，点亮指示灯。 异常处理： 如果收不到 IPB_ESPPATA信号, 按通用策略 10帧认为丢失，点亮指示灯。 备注： N/A |

## 8.3.2 Fault Display Strategy

| Name | Symbol | KL15 Status | HMI Strategy |
| --- | --- | --- | --- |
| ESC Fault Lamp |  | ON | -与ESC fault Lamp共用 仪表显示策略： 1. 如果收到 IPB_ESPFault = 0x0:Notfault，不点亮指示灯。 2. 如果收到 IPB_ESPFault = 0x1:Fault，点亮指示灯； 异常处理： 如果收不到 IPB_ESPFault信号1 按通用策略10 帧认为丢失,点亮指示灯。 备注： N/A |

# Special Performance Requirements

- This part is defined in VW《ESC Performance Acceptance Specification》

# Appendix A

## Change History

| Version | Change Content | Date（D/M/Y） | Reviewer |
| --- | --- | --- | --- |
| V1.0 | Initial edition | 7/9/2025 | Wen Yongzhe |